SPECIFICATIONS

IP CAMERA MODULE

2MP

H.265 / H.264

1. Block Diagram

2. Module Dimension

3. Connectors

| J4 : DAY & NIGHT, Molex P/N : 53047-0210 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | Motor - | Night Mode : Active Low |
| 2 | Motor + | Day Mode : Active High |

| J5 : DC IRIS, Molex P/N : 53047-0410 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | Drive - | Normally connect to GND |
| 2 | Drive + | Iris motor drive |
| 3 | Control + | Iris motor control + |
| 4 | Control - | Iris motor control - |

| J2 : IR_LED, Molex P/N : 53047-0310 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | DC +12V | DC +12V Power output |
| 2 | IR_LED_INPUT | IR_LED CDS signal input from IR board |
| 3 | GND | GND |
| 4 | IR_LED ON/OFF OUTPUT | IR_LED_On/Off TTL signal output to IR board |

| J6 : ZOOM/FOFUS, Molex P/N : 53047-0810 (Optional) |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | Focus A- | Focus In- |
| 2 | Focus A+ | Focus In+ |
| 3 | Zoom A- | Zoom In- |
| 4 | Zoom A+ | Zoom In+ |
| 5 | Zoom B+ | Zoom Out+ |
| 6 | Zoom B- | Zoom Out- |
| 7 | Focus B- | Focus Out- |
| 8 | Focus B+ | Focus Out+ |

| J1 : Power & Network, Molex P/N : 53047-0610 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | +12V | DC +12V input |
| 2 | PHY_TX+ | Ethernet TX+ signal output |
| 3 | PHY_TX- | Ethernet TX- signal output |
| 4 | PHY_RX+ | Ethernet RX+ signal input |
| 5 | PHY_RX- | Ethernet RX- signal input |
| 6 | GND | GND |

| J9 : SD, Molex P/N : 53047-1110 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | DAT2 | SD data2 |
| 2 | DAT3 | SD data3 |
| 3 | CMD | Command signal |
| 4 | PWR | Power +3.3V |
| 5 | CLK | Clock signal |
| 6 | GND | GND |
| 7 | DAT0 | SD data0 |
| 8 | DAT1 | SD data1 |
| 9 | CD | Card Detect signal |
| 10 | WP | Write Protection signal |
| 11 | FACTORY SWITCH | Factory reset signal input signal |

| J8 : AUDIO/ALARM, Molex P/N : 53047-0510 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | AUDIO_IN | Audio Line_In signal |
| 2 | AUDIO_OUT | Audio Line_Out signal |
| 3 | GND | GND |
| 4 | ALARM_IN | Alarm In signal |
| 5 | ALARM_OUT | Alarm Out signal |

| J10 : USB, Molex P/N : 53047-0410 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | VSUB | USB power output DC5V |
| 2 | USB_DM | USB data - |
| 3 | USB_DP | USB data + |
| 4 | GND | GND |

| J7 : CVBS (or UART), Molex P/N : 53047-0410 |  |  |
| --- | --- | --- |
| Pin No. | Pin Name | Description |
| 1 | PHY_LINK | PHY Link indication output signal |
| 2 | GPIO7_1(or UART1_TXD) | GPIO(or UART1_Tx signal) |
| 3 | CVBS_OUT(or UART1_RXD) | Composite output signal(or UART1_Rx signal) |
| 4 | GND | GND |

4. Specifications

| LENS (Optional) |  |
| --- | --- |
| Focal Length | 2.8~12 mm |
| F No. | 1.4±5% |
| Image Format | 1/2.7” inch |
| Dimension(mm) | Φ28.05 x 37.74 x 45.1 mm |
| Field of View | H103° x V74.4° ~ H30.8° x V23.3° |
| IMAGE |  |
| Image Sensor | Sony IMX307LQR-C, 1/2.8” , 2.16Megapixel RGB Bayer Array CMOS Sensor |
| Number of Effective Pixels | 1945(H) x 1097(V) approx. 2.13M pixels |
| Scanning Mode | Progressive Scan |
| CAMERA |  |
| Wide Dynamic Range | DOL HDR (Digital Over Lab High Dynamic Range) |
| Day and Night Mode | Automatic, Day, Night |
| Noise Reduction | 3D-NR |
| Auto Exposure Mode | Automatic, Manual |
| Shutter Speed Control | Automatic: Min. 1/30,000 sec ~ Max. 1/30 sec Manual: Min. 1/30,000 sec ~ Max. 1/30 sec |
| Gain Control | Automatic: Min. 0dB ~ Max. 69dB Manual: Min. 0dB ~ Max. 69dB |
| Auto White Balance Mode | Automatic, Outdoor, Shade, Clear sky, Fluorescent light, Light bulb, Manual |
| Auto Focus (Optional) | One Push, Manual, (Automatic) |
| Optical Zoom (Optional) | Motorized 4.2x |
| Optical Zoom Speed (Optional) | 1~6 Steps |
| Digital Zoom | 8x |
| On Screen Display | On/Off |
| Back Light Compensation | On/Off |
| High Light Compensation | On/Off |
| Image Effect | Mirror, Flip |
| Privacy Mask | Max.8 Area Selection |
| Tamper | On/Off |
| Video Motion Detection | Max.4 Area Selection |
| Corridor Format | 90°/270° |
| Lens Distortion Compensation | On/Off |
| NETWORK |  |
| Video Compression | H.265 Main profile, H.264(High, Main), MJPEG |
| Video Resolution | 1920x1080, 1280x720,1024x576, 768x432, 704x392, 640x360, 384x216 |
| Video Frame Rate | Up to 30fps@1920x1080p |
| Video Streaming | Simultaneously H.265 and H.264 and MJPEG (Quad Streaming) Independent Frame Rate and Bandwidth Control, VBR/CBR Mode |
| Audio Streaming | Two way(G.711) |
| FTP Uploading | MJPEG Still Image |
| Event In | Onboot/Trigger/Network Loss/Tampering/Motion |
| Event Out Notification | E-mail, FTP |
| Login Authority | Administrator, Operator, Guest |
| Event Buffering | FTP : Pre/Post Event Time: 0~30sec, FPS: 1~2 fps |
| Manual Trigger | Still Image Capture |
| Security | Multi User Authority, IP Filtering, HTTPS |
| Network Time Sync | Synchronize Computer/NTP Server, Manual |
| Software Reset | Restart, Reset, Factory Default |
| Auto Recovery | Backup, Restore |
| Remote Upgrade | Using Web Browser |
| Protocols | TCP/IP, UDP, IPv4/v6, HTTP, HTTPS, FTP, UPnP, RTP, RTSP, RTCP, DHCP, ARP, Zeroconf, Bonjour, SNMP |
| Client Software | Built-In Web, ONVIF Compatible 3rd Party VMS |
| Max. User Connection | 10 Users |
| SDK Support | API, ONVIF Profile S/T Compliant |
| EXTERNAL IN/OUT |  |
| Ethernet Tx/Rx | RJ-45(10/100Base-T) |
| IR_LED Control | IR_LED Control On/Off |
| Day & Night Control | IR_Cut_Filter Exchange |
| Iris Control | DC-Iris, Fixed |
| Alarm Control | Alarm-In 1/Alarm-Out 1 |
| SD Card | SDHC/SDXC |
| Audio | Line-In/Line-Out |
| USB | USB 2.0 |
| CVBS(or UART) | Composite Out(or Serial TX/RX) |
| Zoom Control | Max. 4.2x |
| Focus Control | One-Push, Manual, (Automatic) |
| OPERATION ENVIRONMENTS |  |
| Operating Humidity | 0~90% (non-condensing) |
| Operating Temperature | 0℃~ +50℃ |
| Power Supply | DC12V |
| Power Consumption | 4W(without IR_LED) |
| Dimension | 42.0(W) x 42.0(D) x 7.3(H)mm |
| Weight | 15g |

※ Specifications are subject to change without notice in advance.